Manual activPAL algorithm

Aim

This algorithm has been developed to automatically identify wake and bed times in 24 h activPAL data on  an  individual  level  on  multiple  days up to 7 days in total  (i.e.,  different  wake  and  bed  times,  for  each  day,  for  each participant).

The accuracy of this algorithm was examined by comparing wake and bed times estimated by the algorithm with self-reported wake and bed times, using a subsample of The Maastricht Study. It has been shown that the algorithm achieved high levels of agreement.1  A description of the algorithm

can be found elsewhere.1

Content

This manual provides directions on how to run the algorithm to generate wake and bed times as well as  several  activity  parameters.  A  full  description  of  all  parameters  can  be  found  in  the  dictionary Maastricht Study Dictionary Accelerometry Variables MATLAB.xls.

In brief, these include:

o  Wake and bed times

o  Waking time

o  Sleep time/”time in bed”

o  Sedentary time, sedentary bouts, sedentary breaks, sedentary transitions

o  Stand time, stand bouts

o  Stepping time, step bouts

o  Activity score, METs score

o  Step count

o  Stepping intensity

Further wake and bed times  could  manually  be  altered.  Directions on how to make corrections in wake and bed times are provided in this manual.

Finally, an overview on additional options is provided at the end of this manual.

Step 1

The algorithm runs in MATLAB®  (MathWorks. Natick, MA, USA), and uses the .pal or Events.csv files. This program is made with release 2012a and is tested up to version release 2018b.

Download all files and folders (5 folders: backup_wut_bt (empty), data (empty), excel (empty), figures

(empty), matlab (contains files)). Save all .pal or Events.csv files in individual folders within the data folder.

From the matlab folder, run  start_analyse_activepal_nieuw_uitbreiding.m.  This  Graphical  User  Interface  (GUI)  will  appear

(Figure 1).

Figure 1: Graphical User Interface (GUI).

Step 2

Single file

To analyse a single file, choose [input file],   select   the   file,   and   choose [analyse  file].  A figure  will  appear  (see Figure 2) and will be saved ([…].fig; folder: figures).  The generated  wake  and  bed times and the parameters will be saved in an  Excel  file:  uitvoer.xls  (folder:  excel). Each time when a single file is  analysed, the  generated  wake/bed  times  and  the parameters will be added to the Excel file uitvoer.xls.

Figure 2: MATLAB figure representing activity, wake and bed time

Note: when running the GUI, all Excel applications should be closed.

The figure can be used to make corrections in wake and bed time. Directions on how to do this are provided in this manual.

Multiple files

To analyse multiple files, choose [auto analyse] and select the folder data with the files. Then choose  a  file  name  and  location  for  the  Excel  that  will  be  generated.  MATLAB  will  automatically analyse  all files  and  save  the output  in  the  Excel file.  The figures  of  all  files  will also be saved ([…].fig; folder: figures).

Note:  when  running  the  GUI,  all  Excel  applications  should  be  closed.  Please  also  close  all  figures before running the GUI.

Note: due to limitations of Excel, at most 1,200 files could be analysed at the same time. This will

last several hours, as >5,000 parameters will be calculated for each file.

The files with limited amount of data will not be analysed. The command window of MATLAB will then show: “file: […] heeft maar 0 analyse dagen”. In the Excel file, the row of the corresponding .pal file will be filled with zeros.

When  the  GUI  cannot  analyse  a  file  due  to the non wear criteria listed below, (for  example  large amounts  of  time  were  spent  in  one  activity  (e.g.  sedentary,  standing,  stepping)),  the  row  of corresponding file in the Excel file will be filled with zeros, but the ID number will be similar to that of the file of the row above.

Non wear criteria:

- No reaction for 12h or more

- Sedentary for more than 7 hours between 08u00 and 18u00

- Active for 7 hours or more starting between 0u00 and 06u00

In both cases (limited amounts of data and measurement errors), MATLAB will create a file that lists the errors: foute_files.mat. This table could be opened by choosing [auto faults]. The table provides information on the type of error by providing the following information:

Limited amounts of data:

“Undefined function ''aant_wear_days_is_nul'' for input arguments of type ''double''”

Measurement error: the information depends on type of error. Measurement errors could best be seen using the activPAL software. Common errors include:

“Matrix dimensions must agree”

“Subscript indices must either be real positive integers or logicals” “Invalid file identifier.  Use fopen to generate a valid file identifier” “Index exceeds matrix dimensions”

“In an assignment  A(I) = B, the number of elements in B and I must be the same” “Attempted to access ind_neer(0); index must be a positive integer or logical”

Wake and bed times corrections

To make corrections in the estimated wake and bed times, for each file a figure (see Figure 3) will be generated and saved ([…].fig; folder: figures). To view the figure, choose [show figure].

Figure 3: MATLAB figure representing activity, wake and bed times.

Each   row   represents   a   day   of   measurement.   For   example   Wed1represents   the   first

Wednesday of the activPAL measurement period.

On the X-axis daytime is presented from 20:00h (-4) on the day before until 04:00h (28) the day  after.  For  example:  Wed1  is  represented  from  Tuesday  20:00h  until  Thursday  04:00h. Consequently, hours 22-28 on Wed1, are similar to hours -4-0 on Thu1.

The red bars represent activity (standing and stepping). Activity periods with a duration less than 1 minute will be expanded with a black bar for a better visualisation.

The purple bars indicate standing time without stepping.

The green bars indicate waking time; the blue bars indicate bed time.

To zoom in: choose [Quarter Day] or [Half Day].

To zoom out: choose [Whole Day].

Corrections in wake and bed times can be made by clicking on a green or blue bar and dropping the  bar  at  a  chosen  point  in  time.  It  is  not  necessary  to  drop  the  green  bars  exactly  at  the beginning of an activity period (red bars)  – MATLAB will automatically set wake time at the first second  of  the  selected  activity period,  which  will  then  be  indicated  as  wake  time  (Figure  4).  A similar procedure applies for bed time – MATLAB will automatically set bed time at the last second

of the selected activity period, which will then be indicated as bed time.

Figure 4: Correction in wake time.

MATLAB  will  automatically  save  the  corrections  in  wake  and  bed  times  (database_wut_[..].mat  or database_bt_[..].mat). An overview of the corrections can be seen by choosing [WUT corrections] or [BT corrections] (Figure 5).

The  corrections  in  wake/bed  times  will  automatically  be  used,  each  time  when  running  the

GUI. Please close all figures before running the GUI. To remove corrections, select and right-click the correction that should be removed (Figure 5).

Figure 5: Correction in wake time.

Additional options

To test whether the GUI generates data as expected, it is possible to analyse test data. Choose [input testdata]  and  [analyse  file].  A  figure  of  the  test  data  will  appear  and  will  be  saved  ([…].fig;  folder: figures).  The  generated  wake/bed  times  and  the  parameters  will  be  saved  in  an  Excel  file  (folder: excel).

Contact information

For questions about the algorithm or matlab toolkit please contact: 
Wouter Bijnens
Telephone: +31 (0)43 – 388 1455
Email: wouter.bijnens@maastrichtuniversity.nl

For questions or suggestions to improve our GUI, please contact Annemarie Koster: Annemarie Koster, PhD.

P.O. Box 616 – 6200 MD Maastricht – the Netherlands

Telephone: 0031 43 388 2265

Email: a.koster@maastrichtuniversity.nl

References

1.	van der Berg JD, Willems PJB, van der Velde JHPM, Savelberg HHCM, Schaper NC, Schram MT, et al. Identifying waking time in 24-h accelerometry data in adults using an automated algorithm. J Sports Sci. 2016;2:1–7.